安装使用说明书
(1)安装使用说明书是为本样本所有泵类产品而编辑的安装使用指南。

(2)设备应安装在平稳而固定的机架上或水泥地面上，底座的固定螺丝可用膨胀螺丝，大型设备可用底脚螺丝，本系列具有良好的动平衡和无噪音设计。

(3)安装完成后在接管线时建议在进出口安装一段金属波纹软管，以防机头负压变形不能正常工作。

(4)电机采用国内优质电机，电源380V，三相，50H z。泵类系列设备进口在侧面，出口为上面，配对法兰标准为

HG5010-58，PN=0.6Mpa。

(5)在接通电源后启动电机时，如果密封形式是单面密封，那首先先进物料后点动几下电机，在确认物料进入泵腔后，而且无异常响声时，可以正常启动电机；如果如果密封形式是双面密封，必须先通冷却水（油），冷却管在泵头与机座中间的2根细管，进出水方式：为下进上出，待冷却水（油）溢出后，点动几下电机，无异常响声时可以正常启动电机。

(6)在点动电机时一旦发现电流过大，跳闸，电机运转不正常时，首先检查泵头工作腔内是否有异物，或者用手盘动电机是否能转动，如果能转动，就要考虑电机及配电箱接线螺丝是否松动，电线是否符合规定等，以及是否关小闸门减少物料的流量。

(7)严禁石块，铁钉，木块等硬质物料进入泵体，必要时请在进出口处添加快开过滤器。

(8)在处理量大时或扬程高时，请在出口处增加一台相匹配泵作为输送，本系列设备进入口压力≤0.2 Mpa。

(9)较长时间不工作时，请将泵内物料排除干净，排污口在泵头的下面，检修安装时首先擦干净泵体内的每一个要件，然后逐件按顺序安装，完成后首先用手转一下电机，仔细听一下泵体内是否有异常摩擦声，如有请重装，直至无异常声响为止。

本说明书最终解释权属启东市东盛化工机械厂

Installation Instruction
(1) Installation Instruction is edited for all pump products on our catalogue.
(2)Equipment should be installed in the stable and fixed rack or a concrete floor, the base of the fixed screw can be expansive screw. Large-scale equipment can use foot screw.These series have a good dynamic balance and noise-free design.

(3)When take over the line after installation,we suggest that install a metal corrugated hose in the inlet and outlet, which prevents deformation of the nose suction from being not work..
(4)Electric motor adopts superior quality,power supply 380V,three phase,50HZ. Pump series equipment imports in the side,exports above the matching flange standard.HG5010-58, PN = 0.6Mpa.

(5)After connecting to the power the motor get started ,if the design of sealing is singel ,after putting into material and before starting the motor,It should make sure that the materials have been entered the pump chamber and no abnormal noise .If the design of sealing is double ,first passing cooling water (oil),cooling the two tubules between pump head and base .The way of inputing water and outputing water : inputing water from the bottom and outputing the water from the top ,after the overflowing of cooling water (oil),jop the motor seveal times ,if there is no abnormal noise ,the motor can be started normally !

(6)While joping the motor ,if the current become big ,under voltage trip and the motor works unnormally ,firstly check whether there are some foreign body in the working chamber ,then check the motor and find whether it can move by hands ,if it can move ,then you need consider the motor ,the connecting screw of distribution box , and whether the wire is up to specification and try to close the gate little to reduce the flow of the materials .

(7)Stones,nailes,wood such hard materials are prohibited into pump body,if necessary,please add up open filters at extrance and exit.
(8)In large quantity or high lift, please add a match pump to transport at the exit; the inlet’s pressure of this series equipment ≤0.2 Mpa.

(9)Longer period of time when not working,please rule out a clean pump materials,below the outfall in the pump head,repair the installation first wipe every element of the pump body, then piece by piece in order to install, after that turn the motor by hand, Listen carefully if it has unusual friction sound in the pump,if any, please reinstal until no abnormal sound up.

Regarding the above instruction , Qidong dongsheng chemical machinery factory has the final y right to interpret.

